Made in Denmark!

Importer: Ltd “Abschliff Latvija”

Riga, A Čaka iela 107 K2

Phone: +371 67843844, 67843845

Mob.: +371 26177888

E-mail: info@abschliff.lv

www.abschliff.lv

#### Open: Monday to Friday – 9ºº-18ºº, Saturday- 10ºº-14ºº, Sunday  –  closed

WOCA Oil Care – water-based revitaliser for oil-treated wood

Description: Oil Care is a waterbased and indoor climate friendly maintenance oil that can be used on all oiled wood surfaces, oxidating as well as UV-/factory-oiled, and is especially suitable for the regular maintenance of oiled wood floors. Advantages:

- Simple application both manually and by machine;

- Fast-drying;

- Water-based maintenance product;

- Indoor climate certified.

Application area: For all oiled wood surfaces - whether they are finished with oxidative oil or UV oil (factory-oiled). Oil Care Natural is suitable for dark wood species, whereas Oil Care White is used for lighter, possibly lye treated wood such as pine, ash or oak, in order to preserve and enhance the light look.

Treatment:

- Clean the floor with Intensive Wood Cleaner mixed into water. Leave the floor to dry for at least 8 hours. The floor must be completely dry	befor treatment.

- Product, room and floor temperature should be between 15-30°C. Ensure good ventilation at all times during the application and drying time.

- Always remember to test on a less visible spot to check the compatibility of the surface with the product.

- Shake the container carefully.

- Polish in a thin layer of the Oil Care with a white pad, cotton cloth or use polishing machine for large surfaces. It is important that the Oil Care 	is  carefully polished into the wood. Continue polishing until the wood apperas saturated and the surface looks uniform.

- The floor should not appear wet and there should not be any excess oil left on the surface after polishing.

- Work in smaller areas one step at a time until the floor is finished.

Drying time: 12-24 hours at 20°C.

Light use: 3-4 hours at 20°C.

Recommendation: Do not expose the floor to water during hardening time.

Fully hard: 3 days at 20°C.

Technical data: VOC: 15 g/l, maximum VOC content (A/i (WB)): 140 g/l. Density: 0.95-1.00 g/ml. pH-value: pH 8-9. Shelf life: 3 years. Application Temperature: +15-30°C and approx. 50% air humidity. Coverage: 30-40 m²/l. Colours: Natural, White. Cleaning of tools: Use soap and water. Storage:  +10-25°C. Keep out of reach of children. Do not expose to heat (e.g. sunlight). Store frost-free during winter and cool during summer. Packaging 1 l. Viscosity: 20-25 Sec. DIN Cup 4. Odour: Faint. Solid content: 10-20%. Mixing ratio: Ready to use. Do not dilute.

Maintaining and cleaning: For daily cleaning, WOCA Natural Soap is recommended, which effectively washes and cares for the wood. Supplement occasionally with WOCA Oil Refreshing Soap, which also adds a little oil to the wood with each wash.